Narrative Exhibit C:  Need

Milwaukee, WI

Milwaukee_WI_Exhibit_C.doc

EXHIBIT C:  NEED

#### C.1	Severe Physical Distress

HACM’s Scattered Sites development includes 24 units in 22 buildings in Milwaukee’s Near North neighborhood, which HACM acquired and rehabbed at various times in the late 1960s and early 1970s to house low-income families. The predominantly wood-frame buildings were originally built between 1894 and 1926, and on average they are 97 years old.

The Scattered Sites buildings are poorly-designed and poorly situated on their sites, with architectural designs and setbacks which stand out in stark contrast to the surrounding neighborhood fabric. The units are small and overly dense, and none of them are accessible to the mobility-impaired. The physical problems at the buildings are numerous – extensive leaks and water infiltration, and a significant mold and mildew problem as a result; old and failing building components; outdated buildings systems that no longer meet code; lack of insulation or other energy-saving measures; major site drainage challenges; etc. The development no longer serves its purpose as safe, decent housing, and as a result, these Scattered Sites are the focus of the demolition and redevelopment described in this HOPE VI application.

The severe physical distress at the Scattered Sites has been well documented. The condition of the Scattered Sites is primarily responsible for HACM’s low physical inspection scores, with problems like the presence of mold/mildew and lack of accessibility noted repeatedly in the inspection reports. This evaluation has been confirmed by the engineering firm, Graef, Anhalt, Schloemer & Associates, Inc., whose certification of severe physical distress appears in Attachment 14.

Major Deficiencies in the Infrastructure and Systems

The 22 buildings that house the Scattered Sites units vary in age, but all are old, with systems, fixtures, and components that are beyond their useful life. The combination of poor design, lower quality building materials, years of deferred maintenance, and the extreme wear and tear by generations of large families have left the buildings in poor shape, with many serious deficiencies:

Leaky roofs: Many roof shingles are either missing or loose, roof soffit material (which is encapsulating lead-based paint) is missing or damaged, and all chimneys are in need of tuck-pointing or complete reconstruction. As a result, water infiltration is a major problem, and the roof decking has become water-damaged. In addition, damaged ridge vents prevent adequate ventilation of the attic spaces.

Other water infiltration: The exterior walls are clad in hardboard siding and brick veneer, porous materials, and there is no weep or drainage system behind them to channel accumulated moisture to the outside of the structure. In addition, the hardboard siding has delaminated. Thus, water gets trapped behind the existing brick veneer, where it has severely degraded the wood-framed wall systems, contributed to a serious mold/mildew problem, and caused some walls to buckle.

Damaged foundations: The gutters and downspouts function poorly and require replacement, resulting in ineffective drainage which has damaged some foundations. In addition, masonry joints are cracked and the foundations have been weakened by the water seepage.  Freeze/thaw cycles have accelerated the foundation cracking, resulting in buckled basement walls, further water infiltration, and vermin infestation. More detail on these severe drainage and water infiltration problems is provided in the next sub-section.

Mold and mildew: The combination of building envelope failures described above have caused serious interior mold and mildew problems throughout the units. These problems have grown to the point that entire wall sections need to be replaced to ensure resident safety.

Outdated windows/doors: Outdated window and door systems require constant repair and pose a safety threat to residents. Most window screens are torn or missing, and the frames are bent. Most bedroom windows are not adequately sized to meet current egress standards and could trap residents in the event of a fire.

Outdated bathrooms and kitchens: Bathrooms and kitchens need total replacement because of failing cabinetry, tile tub surrounds that allow water to penetrate wall systems, inadequate ventilation, cracked and leaking fixtures, deteriorated plumbing, and undersized drain waste and vent piping.

Outdated electrical systems: Electrical service throughout fails to meet current building standards. Inadequate wiring prevents residents from using air conditioners and certain other appliances. The units also lack ground fault circuit interrupter receptacles. The electrical distribution system requires extensive updating and/or complete replacement to bring it up to current building code requirements.

Outdated heating systems: The buildings’ gas-fired, forced-air furnaces are out-of-date, and the minimal layout of the ducting system results in uneven heat distribution. Residents often use their gas-fired stove/ovens to provide supplemental heating, creating a fire hazard as well as a carbon monoxide poisoning risk.

Major Deficiencies in the Project Site

Inappropriate topography, poor soil conditions, inadequate drainage: The sites’ sloping topography has caused serious drainage and erosion problems, resulting in significant damage to the Scattered Sites buildings. Most of the sites are sloped, with yards that slope down towards the buildings, causing storm water to drain directly toward the foundations. The heavy clay soils that are common in the Milwaukee area require that foundations have proper subsurface drainage systems to divert water from the foundation walls, footings and slabs. Great care also needs to be taken in designing above-grade systems that channel surface water/runoff away from the exterior wall. Needless to say, these kinds of effective drainage systems do not exist at the Scattered Sites buildings, nor has it been possible to alter site grading, since the buildings are often within 3-4 feet of adjacent homes.

Most of the Scattered Sites buildings are located towards the middle and back of their building lots, against the berms that retain the rear alleys.  Because there is not enough room to provide a swale and yard drain that would contain the storm water, the water runs directly over the soft-scape and against the foundation walls. This results in soil erosion in the backyards as well as flooding of the exterior foundation walls each time there is a heavy rainfall. The problem is compounded by the downspouts that carry rainwater from the roof and gutter system and dump it right at the foundation walls. In many cases, the gutters are in such disrepair that water simply cascades down the exterior wall before finding its way to the foundation. Drain tiles that should have been installed at the foundation footings either don’t exist or are completely clogged, due to lack of proper backfill and fabric covering after they were initially placed. The resulting accumulation of water along the exterior basement walls has created enough pressure to cause significant cracking and water infiltration, as noted earlier. The problem is exacerbated by Milwaukee’s severe winters:  freezing temperatures cause the upper three feet of soil to expand when the trapped water freezes, thereby causing additional pressure and widening the cracks.  The basement walls of these units were constructed of concrete blocks, which make them even more susceptible to cracking.  In spring, as the freeze/thaw cycle continues, water pours freely through the soil and the cracks and ends up in the basements, most of which lack sump pumps.  Residents keep most of their stored belongings in the basement on pallets to keep the water out.

Deteriorated laterals and sewers: Some 90% of the existing Scattered Sites units were built on the sites of former dwellings. When the Scattered Sites were constructed, sewer and water lines were reconnected to the existing water and sewer laterals that had been terminated at the public sidewalk during demolition of the original buildings. Thus, the connections from the Scattered Sites side of the sidewalk to the City mains buried in the middle of the street are original 2- to 3-foot sections of clay-hubbed sewer pipe and original lead pipe water laterals.  Most of these are more than 100 years old in this part of the city. Because these sections of pipe were not re-laid during the initial Scattered Site construction, they are now failing on a constant basis. The result is frequent low-flow situations or complete backups, with raw sewage backing into the lowest available drain, which is usually a basement floor drain or laundry tub.

Major Design Deficiencies

## Density: The approximate density of the Scattered Sites development is 6.77 units/acre, which is 65% higher than the 4.1 units/acre density in the city of Milwaukee as a whole.

Unit size: Scattered Sites units are nearly 40% smaller than new market-rate rentals in the city of Milwaukee. The average three-bedroom Scattered Sites unit is 1,336 square feet, the average four-bedroom unit is 981 sf , the average five-bedroom unit is 1,167 sf, and the average five-bedroom unit is 1,333 sf.  Three-bedroom Scattered Sites units have a single full bathroom compared with 1½ bathrooms in the average market-rate rental, and four- and five-bedroom units have 1½ bathrooms compared  with 2 or 2½ bathrooms in the average market-rate rental.

Unit configuration: The units are inappropriately configured for families. The galley-style kitchens do not facilitate family gatherings or relaxed meal preparation. Narrow doors and halls are common. Entry door locations are isolated with stairs on both sides of the door making transition from one room to another difficult. None of the Scattered Sites units are accessible, discussed in more detail further below.

Isolation and indefensible space: The Scattered Sites buildings are set back from the street much more deeply than most of the other buildings in the neighborhood, usually double and sometimes triple the distance from the sidewalk compared with nearby buildings. As a result, people walking or driving down the street often perceive the sites to be vacant lots until they get very close. This break in the rhythm of the street frontage feels unbalanced, and creates a sense of uneasiness in approaching pedestrians, who are unsure of what may lie ahead in this “blank” space. Even more unfortunate, however, is the loss of easy interaction between pedestrians and Scattered Sites residents, because the small stoop that also serves their front porch has no connection to the street. Residents become isolated from their neighbors because they lack the normal bridge between the public and the private realm that a front porch with appropriate setback would provide.

The Scattered Sites buildings themselves do not blend in well with their surroundings, but are clearly identifiable as low-income public housing, with architecture that shouts out “poor people live here!” The buildings lack private porches, patios, clearly delineated front or back yards, and other design features that would indicate individual ownership/control and promote stewardship. Parking is limited and remote from the units. The public realm is not defined or easily distinguishable from what should be private space. The lack of clearly defined and defensible perimeters almost precludes the assignment of individual responsibility.

Older duplexes that are original to the neighborhood are top/bottom stacked flats, and where four-family or larger buildings exist in the original neighborhood layout, they do not repeat on a block face and are constructed utilizing traditional designs. By contrast, the clustering of the Scattered Sites creates visual disharmony, and at the same time increases density to levels that are not consistent with the average block. The pattern of development is a disruption to the tradition urban fabric of the neighborhood, where buildings face the street and have a front yard and backyard accessed by an alley. The mismatch between the Scattered Sites units and their surroundings reinforces the residents’ social and economic isolation from other housing, services, and opportunities.

# Significant utility expenses: The Scattered Sites buildings cost far more than they should to operate, to heat and to cool. Valuable resources are being wasted, resources that could be used for other badly-needed services. By way of comparison, at HACM’s recently completed Townhomes at Carver Park and Parklawn Homes, the same bedroom size units are 20% larger than those at the Scattered Sites but consume 65% less energy on a per-unit basis. Energy audits conducted by We Energies (the local gas and electric utility) as well as HACM’s in-house energy analyst found that utility expenses at the Scattered Sites are high due to the following factors:

- Inadequate or non-existent insulation of areas including: exterior wall cavities, attic spaces, sill plates, foundation walls, exterior door areas, heating distribution ducts and domestic hot water pipes;

- Absence of building wrap and flashings at exterior wall surfaces;

- Single-pane glass windows in wood sash or non-thermally broken aluminum frames;

- Missing or damaged storm doors and windows;

- Overall lack of weather-stripping and other weatherization techniques;

- Outdated gas-fired forced-air furnaces with fuel-efficiency ratings of 60-75%, compared with new models with efficiency ratings of over 90%;

- Outdated gas-fired hot water heaters with standing pilots compared with new energy-efficient units with sidewall exhaust and hot surface ignition systems;

- Thermostats with no time-of-day setbacks;

- Residents use of their gas ovens to supplement the furnace heat; and

- High water consumption due to 3.5-gallon flush toilets (in lieu of 1.6 gallon) and to leaking pipes, valves and faucets.

# Inaccessibility:  None of the Scattered Sites units are accessible or even visitable.  HACM has determined that the cost of conversion to achieve compliance would be prohibitive, and that conversion would require reconfiguration that would further reduce the size of unit living space. Most of the sites lack accessible connections to the city sidewalks. The difference in elevation requires up to 12 steps between the city sidewalk and the front porch stairs with an additional 5 or 6 steps up to the porch. The rear entrances usually have one step up from the driveway or service walk into the hall and another step immediately up to the adjacent room, usually the kitchen, making it extremely difficult for mobility-impaired persons even to access the home.

C.2 	Severe Distress of the Surrounding Neighborhood

The 24 Scattered Sites units proposed here for revitalization are located in the Near North neighborhood, in the very heart of Milwaukee’s urban core. This neighborhood (census tracts 47, 63-68, and 84) is densely populated --10,000 persons per square mile vs. 6,178 persons per square mile for the city as a whole – and it is racially segregated, with a 98% minority population. Violent crime is comparatively high, with over twice as many assaults, sex offenses, and homicides per 1,000 residents than the city average. Over 60% of the adult residents living within the Near North neighborhood are not in the labor force (61.2%). Income levels in the neighborhood are very low, with a median household income of $20,464 vs. $32,216 for the city as a whole, and the rate of poverty is an astounding 39%.

In the Near North neighborhood, levels of disinvestment in both property and businesses have been higher than in other parts of the city. Property values in the neighborhood are among the lowest in the city: the average assessed value of a residential home is $57,000, compared to $152,462 for the city as a whole. The neighborhood contains a high level of subsidized housing units – nearly 1,000 -- and only 43% of its residential housing is owner-occupied, compared with 66% for the city as a whole. Approximately 6% of the city’s vacant land is located in this neighborhood. And Milwaukee Economic Development Corporation business loans totaled only $10,000 for the area vs. $63 million for the city – roughly .02% -- while the Neighborhood Improvement Development Corporation investment in the area for home improvements was $328,423, or 12% of the citywide total of $2,725,417.

The neighborhood has experienced the loss of several large employers of blue collar workers, such as Pabst Brewery, A.O. Smith, and Square D, which moved out of the city and abandoned their land and facilities. The former A.O. Smith site, subsequently referred to as Tower Automotive, comprised over 148 acres and 2 million square feet of industrial space. These losses parallel those of the city and state as a whole. Since September 2000, Wisconsin has lost 96,800 manufacturing jobs, with particularly devastating results for black males: nearly half of all black men in Milwaukee (46%) are not in the labor force. Milwaukee’s jobless rate is the 2nd worst in the U.S., behind only Detroit. This loss of jobs has trapped many families in a web of poverty. Milwaukee has the 8th highest rate of poverty among U.S. cities, with almost 1 in every 4 persons living in poverty.

Although the 24 Scattered Sites units are relatively few in number, and HACM does not claim that they are by themselves the primary driver of the disinvestment in the surrounding neighborhood, the fact that they are scattered over an approximately one square mile area rather than clustered together in one building has magnified the impact of their distressed condition. These 24 units, over half of which have been vacant for more than 6 months, and 8 for more than 1 year, stand out by virtue of their poor design and their increasingly dilapidated conditions, impacting a comparatively high number of adjacent properties, blocks, and streets in the Near North neighborhood due to the simple fact that they are scattered over a sizeable area and belong to HACM. Their revitalization, too, could have a similarly magnified impact.

Despite its signs of physical decay and other problems, the Near North area does possess a strategically advantageous location and is surrounded by a number of hopeful reinvestment projects. To the east is the Riverwest neighborhood, which is emerging as an area of renewal. To the west is the 30th Street Corridor, recently identified by the City as an exciting “green light” district, where the City plans to redevelop the once thriving manufacturing zone by recruiting “green” industries. To the south is North Avenue, which the City is targeting as a historically African American cultural and entertainment center. Numerous Business Investment Districts, Tax Incentive Districts, and Tax Incentive Neighborhoods are located in this area or adjacent to it, focusing planned investments of various kinds.

C.3 	Need for HOPE VI Funding

It will be very difficult to undertake the proposed revitalization activities without a HOPE VI grant. HACM’s Capital Fund Program grant has decreased in recent years: the 2007 grant is more than $1 million less than the amount received in 2003. CFP funding is spread thinly over many competing priorities, including fire sprinkler systems and roofs at HACM’s 11 high-rise buildings. HACM has over a total of $185 million of needed physical improvements, based on a 3rd party assessment. Information about the 2003-2007 CFP grants is provided below.

|  | Budget | $ Obligated | $ Earmarked | $ Available |
| --- | --- | --- | --- | --- |
| 2007 CFP | $6,870,503 | $2,865,631 | $3,560,000 | $444,872 |
| 2006 CFP | $7,401,869 | $3,675,012 | $3,726,857 | $0 |
| 2005 CFP | $7,833,037 | $7,833,037 | N/A | $0 |
| 2004 CFP | $7,480,345 | $7,480,345 | N/A | $0 |
| 2003 CFP set-aside | $1,502,668 | $1,502,668 | Grant closed | $0 |
| 2003 CFP | $6,922,506 | $6,922,506 | Grant closed | $0 |
| Total | $38,010,928 | $30,279,199 | $7,286,857 | $444,872 |

As this table indicates, there is less than $500,000 of available (unobligated and un-earmarked) 2003-2007 CFP funds, which represents less than 10% of the $6.7 million requested for Scattered Sites.

C.4	Previously-Funded Sites

None of the units in this Scattered Sites proposal were included in any previous HOPE VI grant. A list of the Scattered Sites included in this proposal and the 92 Scattered Sites previously funded is in Attachment 23.

C.5	Need for Affordable, Accessible Housing in the Community

There is a tremendous need for affordable housing in the community. Based on 4/30/08 data, HACM’s current public housing occupancy rate is 96% (174 of 3,881 units managed by HACM, excluding the 24 Scattered Sites, were vacant), and HACM’s HCV utilization rate is 100% as of 3/31/08, calculated as expenditures of $6,917,367 out of total authorized funds of $6,929,043.